IoT Education Kit

Lab 2 - Digital I/O and GPIO

Overview

In this lab, we will learn how to configure and use the General Purpose Input/Output (GPIO) capability of a development board, at the register level without the use of an API.

At the end of the lab, you should understand how the configuration of GPIO peripherals is represented at a low level, as well as how to use this to implement simple I/O functionality.

Requirements

In this lab, we will be using the following hardware:

- DISCO-L475VG-IOT01A embedded board

Creating the CMSIS Solution

Follow the instructions for Lab 1 to create a new CMSIS Solution.  This time, call it something like Arm-IoT-Lab2.  As in Lab 1, base it on the “Blinky” example.

Working with the GPIO Peripheral

Configuration via registers

As the GPIO pins are, by name and nature, general purpose, to use them we must specify how we wish the board to treat them. This, as well as additional configuration, is done via a specified set of registers, the values of which correspond to different configurations.

We can write to these registers directly in our program, by treating them as if they were numerical variables. We will show this below with an example where we write to PERIPH_REG, which denotes some register REG associated with some peripheral PERIPH.

Bitwise operations: As with the += and -= (addition/subtraction assignment) operators, X #= Y corresponds to X = X # Y, where # can be any of the bitwise logical operators on two variables, most notably AND and OR, denoted by & and | respectively.

We can also use ~, bitwise NOT, and left and right shifts: << and >>. From this we have the following examples of reading/writing to PERIPH_REG :

| *PERIPH_REG = 0x04; | Set bit[2], and clear all other bits. |
| --- | --- |
| *PERIPH_REG = ~0x04; | Clear bit[2], and set all other bits. |
| *PERIPH_REG ¦= 0x05; | Set bit[0] and bit[2], and leave all other bits unchanged. |
| *PERIPH_REG &= ~0x05; | Clear bit[0] and bit[2], and leave all other bits unchanged. |
| X = !!(*PERIPH_REG & (1 << 2)) | Copy the value of bit[2] to the variable X |

Note: We need to use the * operator to dereference the pointer in PERIPH_REG.

GPIO Registers

Standards used for inputs, outputs, and the associated registers vary between different families of boards, and reference manuals can usually be found through the manufacturers’ websites. For instance, the reference manual for the STM32L475VG family, can be found here:

https://www.st.com/resource/en/reference_manual/rm0351-stm32l47xxx-stm32l48xxx-stm32l49xxx-and-stm32l4axxx-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

Before being able to configure the GPIOs, we have to enable the clock through the corresponding port, by writing to the correct register of the Reset and Clock Control module (RCC).

- RCC AHB2 peripheral clock enable register: AHB2ENR

Most of the modules are disabled by default to save power. Writing a 1 to this register will enable the corresponding clock. Writing a 0 will disable the clock.

Then, the GPIO port has 10 32-bit registers to configure every pin.

- GPIO port mode register: GPIOx_MODER

This register configures the I/O direction mode. Writing into this register will set the corresponding pin as follows:

- 00: General purpose input mode.

- 01: General purpose output mode.

- 10: Alternate function mode.

- 11: Analog mode (reset state).

- GPIO port output type register: GPIOx_OTYPER

This register is used to select between an open drain or pull-up/pull-down resistor configuration for the outputs. Writing a 1 will select the open-drain mode.

- GPIO port output speed register: GPIOx_OSPEEDR

This register can be written to, to select four different speeds for the outputs. Lower speeds improve the power efficiency but may not be suitable for some purposes. Write 00 for low speed, 01 for medium speed, 10 for fast speed, and 11 for high speed.

- GPIO pull-up/pull-down register: GPIOx_PUPDR

This register is used to choose between a pull-up or pull-down resistor, if you haven’t selected open-drain mode in register GPIOx_OTYPER. Write 01 to select the pull-up resister or 10 to select the pull-down. You can also select 00 for no pull-up/down resistor.

- GPIO port input data register: GPIOx_IDR

This register is read only and is used to read input values. These always reflect the logic level of the corresponding pin.

- GPIO port output data register: GPIOx_ODR

This register can be read and written. We can use it to read the current state of the outputs, or to actually set the outputs to high or low. However, for stability, it is recommended to read the inputs through register GPIOx_IDR and set and clean the outputs using register GPIOx_BSRR.

- GPIO port bit set/reset register: GPIOx_BSRR

This register has been separated into two parts. Bits 0 to 15 (the low bits) are set bits, and bits 16 to 31 are the reset bits.

- GPIO port configuration lock register: GPIOx_LCKR

This register is used to lock the configuration of the port bit.

- GPIO alternate function low register (GPIOx_AFRL) and high register (GPIOx_AFRH)

These two registers are used to select the different alternative functions that some pins have.

Register Addresses

All the registers are mapped into the 32-bit global memory space.  In order to be able to read and write from a specific register, we need to know the base address of the peripheral, and the corresponding offset of the register from that base address.

The base addresses for GPIO are listed below:

| Peripheral | GPIOA | GPIOB | GPIOC | GPIOD | GPIOE | GPIOH |
| --- | --- | --- | --- | --- | --- | --- |
| Base Address | 0x48000000 | 0x48000400 | 0x48000800 | 0x48000C00 | 0x48001000 | 0x48001C00 |

Address offsets for the GPIO registers are listed below:

| Register | MODER | OTYPER | OSPEEDR | PUPDR | IDR | ODR | BSRR | LCKR | AFR[2] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offset | 0x00 | 0x04 | 0x08 | 0x0C | 0x10 | 0x14 | 0x18 | 0x1C | 0x20 |

Defining the GPIO Structure

To control a GPIO peripheral, it is best to define the registers into a structure and then create a pointer to it at the corresponding memory base address. Most manufacturers provide the developer with a header file with the addresses and definitions for all the registers, and indeed CMSIS abstracts all of these details away.

However, it’s important to understand what’s actually happening “under the hood”, so we’ll recreate this ourselves.

Open “Blinky.c” in your solution, and replace all of the code with:

#include <stdio.h> int app_main (void) { printf("Low-level Blinky\n"); return 0; }

To make sure everything is working, click the “Build” button, then click the “Run” button, and open the serial monitor and click “Start Monitoring” (if it’s not already).  If the device was programmed and started before you managed to start monitoring, press the “Reset” button on the board, and in the serial monitor you should see “Low-level Blinky” appear.

Now, before the main function definition (int app_main (void)), create the following struct definition:

typedef struct { uint32_t MODER; uint32_t OTYPER; uint32_t OSPEEDR; uint32_t PUPDR; uint32_t IDR; uint32_t ODR; uint16_t BSRRL; uint16_t BSRRH; uint32_t LCKR; uint32_t AFR[2]; } GPIO_PERIPHERAL;

This tells the compiler the layout of the registers for the GPIO_PERIPHERAL, so if we create a pointer to the base address of a GPIO peripheral, we can immediately access the registers.

Let’s do that now, in the main function:

int app_main(void) { printf("Low-level Blinky\n"); volatile GPIO_PERIPHERAL *GPIO_B = (volatile GPIO_PERIPHERAL *)0x48000400; GPIO_B->MODER &= ~(3 << 28); GPIO_B->MODER ¦= (1 << 28); for (;;) { GPIO_B->ODR ¦= 1 << 14; for (unsigned int i = 0; i < 10000000; i++) { asm volatile(""); } GPIO_B->ODR &= ~(1 << 14); for (unsigned int i = 0; i < 10000000; i++) { asm volatile(""); } } return 0; }

There’s quite a bit going on here – so let’s break it down.

This line:

volatile GPIO_PERIPHERAL *GPIO_B = (volatile GPIO_PERIPHERAL *)0x48000400;

Defines a new variable called GPIO_B, which is of type “volatile GPIO_PERIPHERAL *”.  This is a pointer to a GPIO_PERIPHERAL structure (as we defined above), and is marked as volatile so that the compiler doesn’t optimise out any accesses via this pointer.  The volatile keyword is important for I/O accesses, because to a compiler they look useless, but as a programmer, we know they are crucial.

Then, we set the mode of PB14 to be a digital output:

GPIO_B->MODER &= ~(3 << 28); // Clear the two configuration bits for PB14 GPIO_B->MODER ¦= (1 << 28); // Set the output bit for PB14

Following this, there is an infinite loop, which toggles the state of GPIO pin PB14.

This line:

GPIO_B->ODR ¦= 1 << 14;

Sets bit 14 of GPIO B’s Output Data Register.  This bit, when set, brings the PB14 GPIO pin high.  As it happens, PB14 is connected to the green LED next to the USB connector.

This chunk of code:

for (unsigned int i = 0; i < 10000000; i++) { asm volatile(""); }

Is a simple delay loop.  We don’t have the infrastructure set-up to do accurate timing yet, so instead, we just waste time by spinning around in a loop for 10 million iterations.

The asm volatile(“”) directive in the middle of the loop body again prevents the compiler from removing this block of code.  Normally, the compiler would see an empty loop, and get rid of it, because it doesn’t do anything useful – but we really do want to spin around, so the volatile asm statement forces the compiler to keep the loop in.

Finally:

GPIO_B->ODR &= ~(1 << 14);

Clears bit 14 of the GPIO B Output Data Register, thus turning the LED off.  This is followed by another delay loop, so that PB14 toggles regularly.

Working with the CMSIS API

Now that you understand how the GPIO peripherals work, and how they can be accessed at a low-level, we’ll take a look at the CMSIS APIs that are available for controlling GPIOs.

The CMSIS API gives a rich, generic interface to GPIOs, which are implemented by the Board Support Package, which was automatically installed when you created the solution targeting the development board.

To access the GPIO API, add:

#include "GPIO_STM32.h"

To the very top of the file.  You can now call functions from the GPIO driver, e.g.:

Driver_GPIO0.SetDirection(GPIO_PIN_ID_PORTB(14), ARM_GPIO_OUTPUT);

Will change the direction of GPIO pin PB14 to an output.

Exercises

For these exercises, you may need to configure the direction of the GPIO pins accordingly.  Try to do each exercise using low-level I/O and high-level I/O.

Exercise 1 - Blink one LED

You’ve seen above how to do this using raw GPIO access - now use the  Driver_GPIO0 object to blink the green LED attached to PB14.

Exercise 2 - Blink two LEDs alternately

Try using low-level I/O and the high-level Driver_GPIO0 object to blink the green LED attached to PB14 and the green LED attached to PA5 at alternate times (i.e. when one is on, the other is off).

Exercise 3 - Read the state of the USER button, and turn on the LED whilst it is pressed

Try using low-level I/O (Hint: GPIOx_IDR) and the high-level Driver_GPIO0 object to read the state of the USER button attached to PC13, and turn on the LED attached to PB14 when this reads high.

Exercise 4 - Read the state of the USER button, and alternately flash the LEDs whilst it is pressed

As in Exercise 3, use low-level and high-level I/O to read the state of the USER button (PC13), and whilst it is pressed down, the LEDs should alternately blink.